Instructions of the MapInfo_MapMaker software

Video Tutorials link address: http://v.youku.com/v_show/id_XMTcyNzI3MjYw.html

Green software,theoretical deviation is in direct ratio to the visual field diameter. The deviation increases about 1 meter per 1KM distance.

Typical data:If the visual field diameter is 1 km,that the deviation <1 meter ; diameter is 3 km, deviation < 3 meter ; diameter is 10 km, deviation <10 meter.

Note: Before you use this software you should first install the “GoogleEarth” (the following call GE for short), because only when you have open the “GoogleEarth” that you can used this software.

This software has been upgraded four times, the latest version is v1.4.

First open the GE, find the approximate location ,then open this software. Don’t run this software at beginning, because there are some calculations calculate as the basis of “GE have been opened and posited in a lacation”.

The first step, adjust the field of view according to the reference information provided by this software ,then full screen the GE software. Click the "Get information on GE map window" button, you can get some real-time reference information. The red display "except the title bar if the GE is really full-screen outside " is used to determine the full screen state of GE. If it is not full screen ,that you can not register the map,if you click the “register JPG and create TAB and GST” button ,it will pop-up warning message.So you must first full screen the GE ,before any operate.

The “distance between the upper and lower boundary” is used for estimating the effective diameter of the view field , its value is the difference value of the upper and lower boundary’s  Latitude multiplied by the 111000.The unit is meter .Because the latitude are nearly evenly distributed, that is mean the distance is almost the same per degree.

You can adjust the view field according to the actual needs, ，please refer to the “distance between the upper and lower boundary ”.Generally the upper and lower boundary ground distance is not more than 10KM, be in commonly 4KM or so.

After open this software ,you should click the button “obtain GE map window information” to refresh the useful real-time data, included the latitude and longitude of the four vertices which are necessary to register the map,

The second step, using the own screenshots function of GoogleEarth save the map window content as JPG document.. Remember, if there is no change in the map box ,please click the button “obtain GE map window information” .And attention to that the save path of the JPG file cannot on the desktop or has chinese , spaces , strange characters.

The third step,click the button “register JPG and create TAB and GST ”,in the pop-up dialog select the JPG file that you saved in the second step ,then the program will automatic registration of raster file and creat TAB layer files and GST map files.Both the TAB and GST file must save at the same folder with the JPG file and use the same name .If there are TAB and GST file have the same name ,it will directly cover them and have no tips .

The fourth step, calculate the theoretical deviation. Use some software which can open the GST or TAB file and display the latitude and longitude of a point , for example the ground control station software of FY-3ZT autopilot, note the top left vertex and bottom right vertex latitude and longitude (usually format is “ddmm.mmmm”), for example (13338.2234,2245.4321), actual longitude is 133 degrees 38.2234 minutes, the actual latitude is 22 degrees 45.4321 minutes. The software has automatically recorded the latitude and longitude values of the two vertices in the GE and change into the format “ddmm.mmmm”. With the two numerical difference of GE and the MI (such as embedded mapx the ground station software) multiplied by 111,000, that you can get the theoretical deviation of the map that you make by yourself.Theoretical deviation of other point  in the map should between the theoretical deviation of the upper left and lower right vertices.

Illustrated as follows:

1, open the GE, to ensure the"DirectX" and "use safe mode" option in the "Tools -> Options -> 3D View -> Graphics Mode" are both selected, and to ensure that the " topography, " in the sidebar "layer" is not to be checked. That is in order to shut down the GE's terrain attributes,and reduce the latitude and longitude errors caused by terrain ups and downs.

If you want to get the best result, you can close or remove all the irrelevant factors, such as remove the "3D Buildings" and so on..

2, Using the "fly" in the side bar, navigate to the aerial photograph location. Each time the map window changes (such as moving or scaling), you need to click "Get information on GE map window " to refresh the map reference.

PS: If the TAB or GST can not be opened, the problem may be caused here. So you must refresh real-time.

After adjust the view field, minimize all the program cover on the GE, including this software.

3,Close all the menus under the "View", to ensure that the GE really full screen.

If GE is not a full-screen state, will result large deviations in border latitude and longitude information when screenshots, thus affecting registration accuracy. Different versions GE will provide different options of the resolution for saving screenshots. In the free version is only a default screen resolution and do not do tips. In the Professional Edition is a variety of resolution options, select the appropriate resolution and then save the JPG file. Special Note: Do not put the folder in desktop, the path must be a pure English, no Chinese, spaces or strange symbols.

4, Calculated the theoretical deviation.

The ground control station software in the above image is the the FYGCS2.1 software of  Guilin Feiyu Electronic Technology Co., Ltd., through the status bar at the bottom left corner you can see real-time latitude and longitude of the mouse, that is the information of the point where the mouse point to. Move the mouse in the upper left vertex in the map, click on the toolbar zoom tool, as much as possible to enlarge the map, then record the relevant values, and entry to the corresponding position of this software, click the "Calculate", then you can get the theoretical deviation of the upper left vertex ,its unit is “meter”.Use the similarly method to calculate the theoretical deviation the lower right vertex.

5, Added: This is a free software, anyone is free of charge to use it. Any comments or suggestions, please contact E-mail: feiyudz@yahoo.cn.The software may continually upgrade, please refer: http://www.feiyudz.cn .